# Laboratory Activity# 1 (Midterm): Creating Ethernet Cables

Straight-Through, Crossover, and Rollover (Console) Cables

Name:  _________________________				Date: _____________________

Section: ________________

Learning Outcomes (Student Will Be Able To):

After this activity, students should be able to:

- Identify the differences between straight-through, crossover, and rollover cables.

- Correctly arrange UTP Cat6 wires using T568A and T568B standards.

- Terminate Ethernet cables using RJ45 connectors.

- Test cable functionality using a LAN cable tester.

- Apply correct cable types to real networking scenarios.

## Materials Required (Per Group)

- UTP Cat6 cable (at least 2 meters)

- RJ45 connectors (minimum 6 pcs)

- Crimping tool

- LAN cable tester

- Wire stripper / cutter

- Labels or masking tape

## Cable Types Overview

| Cable Type | Standard Used | Common Purpose |
| --- | --- | --- |
| Straight-Through | T568B–T568B | PC to Switch, Router to Switch |
| Crossover | T568A–T568B | PC to PC, Switch to Switch |
| Rollover | Reversed pins | PC to Router Console |

## Wire Color Standards

### T568A

| Pin | Color |
| --- | --- |
| 1 | White/Green |
| 2 | Green |
| 3 | White/Orange |
| 4 | Blue |
| 5 | White/Blue |
| 6 | Orange |
| 7 | White/Brown |
| 8 | Brown |

### T568B

| Pin | Color |
| --- | --- |
| 1 | White/Orange |
| 2 | Orange |
| 3 | White/Green |
| 4 | Blue |
| 5 | White/Blue |
| 6 | Green |
| 7 | White/Brown |
| 8 | Brown |

## Activity A: Making a Straight-Through Cable

### Steps:

- Cut a 1-meter UTP Cat6 cable.

- Strip 1 inch of outer jacket from both ends.

- Untwist wires and arrange them in T568B order on both ends.

- Flatten wires and trim evenly.

- Insert wires into RJ45 (clip facing down).

- Crimp both ends.

- Test using LAN tester (lights 1–8 should be sequential).

Expected Result: Passes continuity test (1→1, 2→2 … 8→8)

## Activity B: Making a Crossover Cable

### Steps:

- Strip both ends of the cable.

- Arrange End A using T568A.

- Arrange End B using T568B.

- Insert into RJ45 connectors.

- Crimp both ends.

- Test using LAN tester.

Expected Result: Pins 1↔3 and 2↔6 are crossed

## Activity C: Making a Rollover (Console) Cable

### Steps:

- Strip both ends of the cable.

- Arrange End A in T568B order.

- Arrange End B in reverse order:

  - Pin 1 → Pin 8

  - Pin 2 → Pin 7

  - Pin 3 → Pin 6

  - Pin 4 → Pin 5

- Crimp both ends.

- Test using LAN tester.

Expected Result: Fully reversed pin sequence

## Testing & Verification Table

| Cable Type | Test Result | Remarks |
| --- | --- | --- |
| Straight-Through | ☐ Pass ☐ Fail |  |
| Crossover | ☐ Pass ☐ Fail |  |
| Rollover | ☐ Pass ☐ Fail |  |

## Documentation Requirement

Students must submit:

- Photo of wire color arrangement before crimping

- Photo of cable tester result

- Photo of finished cables (labelled)

## Grading Rubric ( 50 pts)

| Criteria | Points |
| --- | --- |
| Correct wire order | 15 |
| Proper crimping | 10 |
| Cable tester result | 10 |
| Guided questions | 12 |
| Safety & tool handling | 3 |
| Total | 50 |

## Guide Questions

- Why is T568B commonly used in straight-through cables?

We used T568B for straight-through cables because it’s the most common wiring standard in networks today. Since most devices like PCs, switches, and routers expect this standard, using T568B on both ends makes the connection work properly without issues. It also keeps things consistent and avoids confusion when setting up or troubleshooting the network.

- When is a crossover cable required?

We learned that a crossover cable is required when we directly connect two similar devices, like PC-to-PC or switch-to-switch, without using a router or hub in between. The crossover allows the transmit and receive wires to switch places so the devices can communicate correctly.

- What device uses a rollover cable?

We used a rollover cable when connecting a computer to a network device’s console port, usually a router or a switch. This type of cable is mainly for configuration and management, not for regular data transmission.

- What happens if one wire is misplaced?

When one wire is misplaced, the cable may not work at all or the connection becomes unstable. From what we observed, even a single wrong wire can cause slow speeds, data loss, or complete network failure, which is why proper wire order is really important when crimping cables.