6ITN MIDTERM PERIOD

2nd Semester, School Year 2025-2026

Name: Carosus, Cean A.								Score: __________

Section: CS - 203	Class Code: 2250						Date: 02/13/2026

# Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway | PING STATUS [Y/N] |
| --- | --- | --- | --- | --- | --- |
| R1 | G0/0 | 192.168.100.1 | 255.255.255.224 | N/A | Y |
| R1 | G0/1 | 192.168.100.33 | 255.255.255.224 | N/A | Y |
| R1 | S0/0/0 | 192.168.100.129 | 255.255.255.224 | N/A | Y |
| R2 | G0/0 | 192.168.100.65 | 255.255.255.224 | N/A | Y |
| R2 | G0/1 | 192.168.100.97 | 255.255.255.224 | N/A | Y |
| R2 | S0/0/0 | 192.168.100.158 | 255.255.255.224 | N/A | Y |
| S1 | VLAN 1 | 192.168.100.2 | 255.255.255.224 | 192.168.100.1 | Y |
| S2 | VLAN 1 | 192.168.100.34 | 255.255.255.224 | 192.168.100.33 | Y |
| S3 | VLAN 1 | 192.168.100.66 | 255.255.255.224 | 192.168.100.65 | Y |
| S4 | VLAN 1 | 192.168.100.98 | 255.255.255.224 | 192.168.100.97 | Y |
| PC1 | NIC | 192.168.100.30 | 255.255.255.224 | 192.168.100.1 | Y |
| PC1 to R1 Console |  |  |  |  | Y |
| PC2 | NIC | 192.168.100.62 | 255.255.255.224 | 192.168.100.33 | Y |
| PC3 | NIC | 192.168.100.94 | 255.255.255.224 | 192.168.100.65 | Y |
| PC3 to R1 Console |  |  |  |  | Y |
| PC4 | NIC | 192.168.100.126 | 255.255.255.224 | 192.168.100.97 | Y |

# Objectives

Part 1: Design an IP Addressing Scheme

Part 2: Complete the topology. Assign IP Addresses to all Network Devices including ports and Verify Connectivity

# Scenario

In this activity, you are given the network address of 192.168.100.0/24 to subnet and provide the IP addressing for the Packet Tracer network. Each LAN in the network requires at least 25 addresses for end devices, the switch and the router. The connection between R1 to R2 will require an IP address for each end of the link.

# Instructions

## Design an IP Addressing Scheme

### Subnet the 192.168.100.0/24 network into the appropriate number of subnets.

Note: If your answer is less than the 25 hosts required, then you borrowed too many bits. You may use available PC for console to intermediary devices (eg. switches). Delete icon is enabled. Be careful when deleting.

Calculate the binary value for the first five subnets. The first two subnets have been done for you.

| Subnet | Network Address | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 192.168.100. | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 192.168.100. | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 |
| 2 | 192.168.100. | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| 3 | 192.168.100. | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 |
| 4 | 192.168.100. | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Calculate the binary and decimal value of the new subnet mask.

| First Octet | Second Octet | Third Octet | Mask Bit 7 | Mask Bit 6 | Mask Bit 5 | Mask Bit 4 | Mask Bit 3 | Mask Bit 2 | Mask Bit 1 | Mask Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 11111111 | 11111111 | 11111111 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 |
| First Decimal Octet | Second Decimal Octet | Third Decimal Octet | Fourth Decimal Octet |  |  |  |  |  |  |  |
| 255. | 255. | 255. | 224 |  |  |  |  |  |  |  |

Fill in the Subnet Table, listing the decimal value of all available subnets, the first and last usable host address, and the broadcast address. Repeat until all addresses are listed. *Note: You may not need to use all rows.

Subnet Table

| Subnet Number | Subnet Address | First Usable Host Address | Last Usable Host Address | Broadcast Address |
| --- | --- | --- | --- | --- |
| 0 | 192.168.100.0 | 192.168.100.1 | 192.168.100.30 | 192.168.100.31 |
| 1 | 192.168.100.32 | 192.168.100.33 | 192.168.100.62 | 192.168.100.63 |
| 2 | 192.168.100.64 | 192.168.100.65 | 192.168.100.94 | 192.168.100.95 |
| 3 | 192.168.100.96 | 192.168.100.97 | 192.168.100.126 | 192.168.100.127 |
| 4 | 192.168.100.128 | 192.168.100.129 | 192.168.100.158 | 192.168.100.159 |
| 5 | 192.168.100.160 | 192.168.100.161 | 192.168.100.190 | 192.168.100.191 |
| 6 | 192.168.100.192 | 192.168.100.193 | 192.168.100.222 | 192.168.100.223 |
| 7 | 192.168.100.224 | 192.168.100.225 | 192.168.100.254 | 192.168.100.255 |

### Assign the subnets to the network shown in the topology.

Assign Subnet 0 to the LAN connected to the GigabitEthernet 0/0 interface of R1:

Assign Subnet 1 to the LAN connected to the GigabitEthernet 0/1 interface of R1:

Assign Subnet 2 to the LAN connected to the GigabitEthernet 0/0 interface of R2:

Assign Subnet 3 to the LAN connected to the GigabitEthernet 0/1 interface of R2:

Assign Subnet 4 to the WAN link between R1 to R2:

### Document the addressing scheme.

Fill in the Addressing Table using the following guidelines:

Assign the first usable IP addresses in each subnet to R1 for the two LAN links and the WAN link.

Assign the first usable IP addresses in each subnet to R2 for the LAN links. Assign the last usable IP address for the WAN link.

Assign the second usable IP address in the attached subnets to the switches.

Assign the last usable IP addresses to the PCs in each subnet.

### Assign IP Addresses to Network Devices and Verify Connectivity

### Configure R1 & R2 LAN interfaces.

Configure both LAN interfaces with the addresses from the Addressing Table.

Configure the interfaces so that the hosts on the LANs have connectivity to the default gateway.

### Configure IP addressing on S1, S2, S3 & S4.

Configure the switch VLAN1 interface with addressing.

Configure the switch with the default gateway address.

### Configure all PCs.

Configure all PCs with host and default gateway addresses.

### Configure static routes for each router.

R1: ip route 0.0.0.0 0.0.0.0 [S/0/0 address]

R2: ip route 0.0.0.0 0.0.0.0 [S/0/0 address]

### Verify connectivity.

You should be able to ping every IP address listed in the Addressing Table.

- Sample Device Configurations:

- R1:

- enable

- configure terminal

- interface GigabitEthernet0/0

- ip address 192.168.100.X 255.255.255.X

- no shutdown

- interface GigabitEthernet0/1

- ip address 192.168.100.X 255.255.255.X

- no shutdown

- interface Serial0/0/0

- ip address 192.168.100.X 255.255.255.224

- no shutdown

- end

- enable

- configure terminal

- router rip

- network 192.168.100.XX

- S1:

- enable

- configure terminal

- interface VlanX

- ip address 192.168.100.X 255.255.255.X

- no shutdown

- ip default-gateway 192.168.100.X

- end

- PC1:

- IP address: 192.168.100.30 /27

- Default gateway: 192.168.100.1